MicroServer Web Server Patches

Edited: 09-August-05

There is a patch available that fixes vulnerabilities in the Xerox MicroServer Web Server that has been identified on Document Centre Multifunction Devices (MFD). The patch software only needs to be applied to the MFD if the System Software version of your MFD falls within the range listed.  Patch P25 replaces patch P4 only.

The patch is packaged in a ZIP format. Download the ZIP file from the URL provided and extract all contents to your desktop. The patch should be installed on the devices as specified below. The patch should be sent to the devices as is – do not open the files.

Instructions for the Document Centre 220/230/332/340. P25	2

Appendix A – Enabling LPD, port 515 printing	2

# Instructions for the Document Centre 220/230/332/340. P25

Patch File Name: P25_http_DC220-340.dlm

Required for ESS Versions:

1.12.08 through 1.12.85

If your device has 1.12.87 higher ESS version, you do not need to install the patch.

#### Confirm your ESS Software Version

To determine your ESS Software version, you can either print a Configuration Report or view the version on the Web client interface.

To print a configuration report from the local User Interface at the machine:

- Press the Machine Status button

- Select Print Configuration Report

- Look for the ESS Software Version number

To view the version from the web client interface:

- Open a web browser and connect to the multifunction device by entering the IP number of the device

- Select the “Device Index” icon in the upper middle portion of the screen.

- Select “Device Profile”.

- Scroll to the location that displays the ESS Software Version.

#### Install the Patch

DO NOT TRY TO OPEN THE PATCH AS IT MAY DAMAGE THE FILE.

###### LPR Method from a Windows NT, 2000, or XP

This method requires that LPD Protocol be enabled on the device. Check the configuration report to see if the LPD protocol is enabled.  This protocol can be enabled via the Local User Interface or via the Web Interface. See Appendix A for instructions.

- Open a “DOS Command Prompt”.  You can do this by selecting the Windows “Start” icon, and selecting “Run”.  Type “cmd” and hit <Enter>.

- Submit the patch file via the command line: lpr –S <printer_ip> –P lp P25_http_DC220-340.dlm

- Power the device Off, then On. Wait for device to boot.

- Power the device off then on again.

- The patch is installed when .P25 is appended to the ESS version number.

NOTE: If P25 is not appended to the ESS version number, then you must contact the customer support center to have your machine upgraded to ESS 1.12.85 or the latest available release.

# Appendix A – Enabling LPD, port 515 printing

In order to use the LPR method to submit the patch, your MFD must support Line Printer Daemon (LPD) over port 515. Most MFD’s have this enabled by default. If you have disabled LPD printing, you must enable it to use the LPR method.

For the Document Centre 220/230/332/340 use the following steps to enable LPD:

- Open a web browser and connect to the multifunction device by entering the IP number of the device

- Select “Device Index” icon in the upper right corner

- Select “Protocols”, then scroll to LPD and select the LPD link.

- If the Enabled box is NOT checked, select the box to add a check mark.

- Select “Apply New Settings”

- Enter the user name Admin and the admin password, then select OK.

- Power the MFD off then on.

<End of text>